Microprocessor and Computer Architecture

UE21CS251B

4th Semester, Academic Year 2022-23

Date:

| Name: Nikhil Girish | SRN: PES2UG21CS334 | Section: F |
| --- | --- | --- |

Week#____2_______            Program Number: ____1___

Title of the Program

Write a program in ARM7TDMI-ISA to copy a block of N data items from Location A to Location B.

a. Use Full word (.word directive)

b. Use Half word(.hword directive)

c.  Use Byte wise (.Byte directive)

- ARM Assembly Code:

a:

.data

a: .word 10,20,30,40,50,60,70,80,90,100

b: .word 0,0,0,0,0,0,0,0,0,0

.text

LDR R0,=a

LDR R1,=b

MOV R4,#0

l1:

LDR R3,[R0]

STR R3,[R1]

ADD R0,R0,#4

ADD R1,R1,#4

ADD R4,R4,#1

CMP R4, #10

BNE l1

SWI 0x011

.end

b:

.data

a: .hword 1,2,3,4,5,6,7,8,9,10

b: .hword 0,0,0,0,0,0,0,0,0,0

.text

LDRH R0,=a

LDRH R1,=b

MOV R4,#1

l1:

LDRH R3,[R0]

STRH R3,[R1]

ADD R0,R0,#2

ADD R1,R1,#2

ADD R4,R4,#1

CMP R4, #11

BNE l1

SWI 0x011

c:

.data

a: .byte 1,2,3,4,5,6,7,8,9,10

b: .byte 0,0,0,0,0,0,0,0,0,0

.text

LDR R0,=a

LDR R1,=b

MOV R4,#1

l1:

LDRB R3,[R0]

STRB R3,[R1]

ADD R0,R0,#1

ADD R1,R1,#1

ADD R4,R4,#1

CMP R4, #11

BNE l1

SWI 0x011

- Output Screen Shots  (Three)

The output should be verified for word, half word, byte

Word:

Half-word:

Byte:

Microprocessor and Computer Architecture

UE21CS251B

4th Semester, Academic Year 2022-23

Date:

| Name: Nikhil Girish | SRN: PES2UG21CS334 | Section: F |
| --- | --- | --- |

Week#____2_______            Program Number: ____2___

Title of the Program

Write a program in ARM7TDMI-ISA to find the sum of  N data items in the memory. Store the result in the memory location.

a. Use Full word (.word directive)

b. Use Half word(.hword directive)

c.  Use Byte wise (.Byte directive)

I.ARM Assembly Code

a:

.data

a: .word 10,20,30,40,50,60,70,80,90,100

b: .word 0

.text

LDR R0,=a

LDR R1,=b

MOV R4,#0

MOV R5,#0

l:

LDR R2,[R0]

LDR R3,[R1]

ADD R4,R2,R3

STR R4,[R1]

ADD R0,R0,#4

ADD R5,R5,#1

CMP R5, #10

BNE l

SWI 0x011

b:

.data

a: .hword 10,20,30,40,50,60,70,80,90,100

b: .hword 0

.text

LDR R0,=a

LDR R1,=b

MOV R4,#0

MOV R5,#0

l:

LDRH R2,[R0]

LDRH R3,[R1]

ADD R4,R2,R3

STRH R4,[R1]

ADD R0,R0,#2

ADD R5,R5,#1

CMP R5, #10

BNE l

SWI 0x011

c:

.data

a: .byte 10,20,30,40,50,60,70,80,90,100

b: .byte 0

.text

LDR R0,=a

LDR R1,=b

MOV R4,#0

MOV R5,#0

l:

LDRB R2,[R0]

LDRB R3,[R1]

ADD R4,R2,R3

STRB R4,[R1]

ADD R0,R0,#1

ADD R5,R5,#1

CMP R5, #10

BNE l

SWI 0x011

II. Output Screen Shots (Three)

The output should be verified for word, half word, byte

Word:

Half-word:

Byte:

Microprocessor and Computer Architecture

UE21CS251B

4th Semester, Academic Year 2022-23

Date:

| Name: Nikhil Girish | SRN: PES2UG21CS334 | Section: F |
| --- | --- | --- |

Week#____2_______            Program Number: ____3___

Title of the Program

Write a program in ARM7TDMI-ISA to find the sum of  N natural numbers.  Store the result in the memory location.

I.ARM Assembly Code:
.data

a: .byte 1,2,3,4,5,6,7,8,9,10

b: .byte 0

.text

LDR R0,=a

LDR R1,=b

MOV R4,#0

MOV R5,#0

l:

LDRB R2,[R0]

LDRB R3,[R1]

ADD R4,R2,R3

STRB R4,[R1]

ADD R0,R0,#1

ADD R5,R5,#1

CMP R5, #10

BNE l

SWI 0x011

II. Output Screen Shots (One):

Microprocessor and Computer Architecture

UE21CS251B

4th Semester, Academic Year 2022-23

Date:

| Name: Nikhil Girish | SRN: PES2UG21CS334 | Section: F |
| --- | --- | --- |

Week#____2_______            Program Number: ____4___

Title of the Program

Write a program in ARM7TDMI-ISA to find the product of two 32bit numbers using barrel shifter.

I.ARM Assembly Code:

.data

a: .word -4

b: .word 9

.text

LDR R0, =a

LDR R0, [R0]

LDR R1, =b

LDR R1, [R1]

MOV R2, #0

MOV R3, #0

l:

CMP R0, #0

BEQ end

TST R0, #1

BEQ l1

ADD R3, R3, R1, LSL R2

l1:

MOV R0, R0, LSR #1

ADD R2, R2, #1

B l

end:

SWI 0x011

II. Output Screen Shot  (One):

Microprocessor and Computer Architecture

UE21CS251B

4th Semester, Academic Year 2022-23

Date:

| Name: Nikhil Girish | SRN: PES2UG21CS334 | Section: F |
| --- | --- | --- |

Week#____2_______            Program Number: ____5___

Title of the Program

Convert the following statement in C language into an ALP using ARM7TDMI – ISA.

IF([A]==[B]) then C=[A]+[B];

ELSE IF ([B]==[C]) D=[A]-[B];

ELSE E=[A]*[B]

Where A,B C, D & E are memory locations.

I.ARM Assembly Code:

.data

a: .word 2

b: .word 3

c: .word 4

d: .word 0

e: .word 0

.text

LDR R0, =a

LDR R1, =b

LDR R2, =c

LDR R3, =d

LDR R4, =e

LDR R0, [R0]

LDR R1, [R1]

LDR R5, [R2]

TEQ R0, R1

BEQ l

TEQ R1, R5

BEQ l1

MUL R6, R0, R1

STR R6, [R4]

B end

l:

ADD R6, R0, R1

STR R6, [R2]

B end

l1:

SUB R6, R0, R1

STR R6, [R3]

B end

end:

SWI 0x011

II. Output Screen Shot  (One):

Microprocessor and Computer Architecture

UE21CS251B

4th Semester, Academic Year 2022-23

Date:

| Name: Nikhil Girish | SRN: PES2UG21CS334 | Section: F |
| --- | --- | --- |

Week#____2_______            Program Number: ____6__

Title of the Program

Write a program in ARM7TDMI-ISA to find the factorial of a number.

I.ARM Assembly Code:

.data

a: .word 7

.text

LDR R0,=a

LDR R0, [R0]

MOV R1,#1

l:

MUL R1,R0,R1

SUBS R0,R0,#1

BNE l

SWI 0x011

II. Output Screen Shot  (One):

Disclaimer:

- The programs and output submitted is duly written, verified and executed by me.

- I have not copied from any of my peers nor from the external resource such as internet.

- If found plagiarized, I will abide with the disciplinary action of the University.

Signature:

Name: Nikhil Girish

SRN: PES2UG21CS334

Section: 4F

Date: 28-01-2023